Duet Module Interface Specification

for the

DirecTV HR21

Digital Satellite System (DSS)

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	4

Channels	6

Levels	7

Command Control	8

Command Feedback	10

Device Notes	11

Programming Notes	11

Adding Functions to Modules	12

Commands to the device	12

Responses from the device	12

Revision History

| ###### Date | Initials | Version | Comments |
| --- | --- | --- | --- |
| 09-10-08 | SP | V1.0.0 | Initial release |
| 01-20-09 | SP | V1.0.1 | Added a new level for Signal Quality |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a DirecTV HR21. The device supports an RS-232 serial protocol. The interface was tested using version 1.6 of the firmware. The required communication settings are a baud rate of 9600, 8 data bits, 1 stop bit, no parity, and handshaking off. The cable for this device is FG#10-752-04. The wiring diagram for this cable is as follows:

AMX NXI	AMX NI 		Device (Male DB9)

(Gnd) 1	(Gnd) 5  __________	(Gnd) 5

(Rx)   2	(Rx)   2  __________	(Tx)   2

(Tx)   3	(Tx)   3  __________	(Rx)   3

This module was written using Café Duet firmware version 3.30.371, NetLinx Studio version 2.7 build 2.7.0.226, Café Duet application platform and runtime version 1.10.203, and Café Duet application plug-in version 1.8.85.

## Overview

The COMM module translates between the standard interface described below and the DirecTV HR21 serial protocol. It parses the buffer for responses from the DirecTV HR21, sends strings to control the DirecTV HR21, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

DirecTV HR21

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

Some functionality in the device interface may not be implemented in the API interface.  In cases where device functions are desired but not API-supported, the PASSTHRU command may be used to send any and all device-protocol commands to the device. See the PASSTHRU command and the Adding Functions to Modules section for more information.

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

## Implementation

To interface to the AMX DirecTV HR21 module, the programmer must perform the following steps:

- Define the device ID for the DirecTV HR21 that will be controlled.

- Define the virtual device ID that the DirecTV HR21 COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000.

- If a touch panel interface is desired, a touch panel file DirecTV HR21.TP4 and module ( DirecTV HR21 Main.axs) have been created for testing.

- The Duet DirecTV_HR21_Comm_dr1_0_0.jar module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the DirecTV HR21 to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

vdvDirecTVHR21 = 41001:1:0  // The COMM module should use this as its duet device

dvDirecTVHR21 = 5001:1:0 // This device should be used as the physical device by the COMM module

dvDirecTVHR21Tp = 10001:1:0

DEFINE_VARIABLE

DEV vdvDev[] = {vdvDirecTVHR21}

// ----------------------------------------------------------

// CURRENT DEVICE NUMBER ON TP NAVIGATION BAR

INTEGER nDirecTVHR21 = 1

// ----------------------------------------------------------

// DEFINE THE PAGES THAT YOUR COMPONENTS ARE USING IN THE

// SUB NAVIGATION BAR HERE

INTEGER nTunerStationPages[] = { 1,2 }

INTEGER nMenuPages[] = { 3,4,5,6 }

INTEGER nPowerPages[] = { 7 }

INTEGER nModulePages[] = { 8 }

DEFINE_START

// ----------------------------------------------------------

// DEVICE MODULE GROUPS SHOULD ALL HAVE THE SAME DEVICE NUMBER

DEFINE_MODULE 'DirecTV HR21 MenuComponent' menu(vdvDev, dvDirecTVHR21Tp, nDirecTVHR21, nMenuPages)

DEFINE_MODULE 'DirecTV HR21 ModuleComponent' module(vdvDev, dvDirecTVHR21Tp, nDirecTVHR21, nModulePages)

DEFINE_MODULE 'DirecTV HR21 PowerComponent' power(vdvDev, dvDirecTVHR21Tp, nDirecTVHR21, nPowerPages)

DEFINE_MODULE 'DirecTV HR21 TunerStationComponent' tunerstation(vdvDev, dvDirecTVHR21Tp, nDirecTVHR21, nTunerStationPages)

DEFINE_MODULE 'DirecTV HR21 DiscTransportComponent' disctransport(vdvDev, dvDirecTVHR21Tp, nDirecTVHR21, nPowerPages)

// Define your communications module here like so:

DEFINE_MODULE 'DirecTV_HR21_Comm_dr1_0_0' comm(vdvDirecTVHR21, dvDirecTVHR21)

Upon initialization the AMX Comm module will communicate with the DirecTV HR21 and information will be exchanged.

## Channels

The UI module controls the DirecTV HR21 via channel events (NetLinx commands pulse, on, and off) sent to the COMM module. The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

| Channel | Description |
| --- | --- |
| 1 | PULSE: Play |
| 2 | PULSE: Stop |
| 3 | PULSE: Pause |
| 6 | PULSE: Scan Forward |
| 7 | PULSE: Scan Reverse |
| 8 | PULSE: Record |
| 9 | PULSE: Cycle Power |
| 10 | PULSE: 0 Digit Button |
| 11 | PULSE: 1 Digit Button |
| 12 | PULSE: 2 Digit Button |
| 13 | PULSE: 3 Digit Button |
| 14 | PULSE: 4 Digit Button |
| 15 | PULSE: 5 Digit Button |
| 16 | PULSE: 6 Digit Button |
| 17 | PULSE: 7 Digit Button |
| 18 | PULSE: 8 Digit Button |
| 19 | PULSE: 9 Digit Button |
| 21 | PULSE: Enter Button |
| 27 | PULSE: Set Power On |
| 28 | PULSE: Set Power Off |
| 44 | PULSE: Menu Button |
| 45 | PULSE: Move Menu Cursor Up |
| 46 | PULSE: Move Menu Cursor Down |
| 47 | PULSE: Move Menu Cursor Left |
| 48 | PULSE: Move Menu Cursor Right |
| 49 | PULSE: Select Menu Item |
| 50 | PULSE: Exit Button |
| 81 | PULSE: Back Button |
| 86 | PULSE: List Button |
| 90 | PULSE: Dash Button |
| 101 | PULSE: Info Button |
| 105 | PULSE: Guide Button |
| 225 | PULSE: Increment Station |
| 226 | PULSE: Decrement Station |
| 235 | PULSE: Go to previous station |
| 241 | ON: Play is active – provides feedback only |
| 242 | ON: Stop is active – provides feedback only |
| 243 | ON: Pause/Record Pause is active – provides feedback only |
| 246 | ON: Scan Forward is active – provides feedback only |
| 247 | ON: Scan Reverse is active – provides feedback only |
| 248 | ON: Record is active – provides feedback only |
| 251 | ON: Device is Online – used for feedback only OFF: Device is not Online |
| 252 | ON: Data is Initialized – use for feedback only OFF: Data is not Initialized |
| 255 | ON: Set power on – used for feedback also OFF: Set power off |

Table 1 - Virtual Device Channel Events

## Levels

The UI module controls the DirecTV HR21 via level events (NetLinx command send_level) sent to the COMM module. The levels supported by the COMM module are listed below. These levels are associated with the virtual device(s) and are independent of the levels associated with the touch panel device.

Note: An ‘*’ indicates an extension to the standard API.

| Level | Description |
| --- | --- |
| *33 | Signal Quality Level (range 0…100) – Feedback Only |

Table 2 - Virtual Device Level Events

## Command Control

The UI module controls the DirecTV HR21 via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: An ‘*’ indicates an extension to the standard API.

| Command | Description |
| --- | --- |
| ?DEBUG | Request the state of the debug feature. ?DEBUG |
| DEBUG-<value> | Set the state of debugging messages in the UI module and the Comm. module. Note: See Programming Notes section. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning & info messages on 4 = set all messages on DEBUG-1 |
| ?FWVERSION | Get the device’s firmware version. ?FWVERSION |
| PASSBACK-<state> | Enable or disable response reporting from the device. When enabled device responses will be sent as strings to the virtual device. Note: By default, this is set to off at startup. <state> : 0 = Off (default) 1 = On PASSBACK-0 |
| PASSTHRU-<string> | Allows user the capability of sending commands directly to whatever unit is attached with minimal processing by the Duet module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features that may not be directly supported by the module. For more information, see the “Adding Functions to Modules” section below. <string> : string to send to unit “’PASSTHRU-‘,$FA,$A5,$00,$01,$D3” (Guide) |
| ?PROPERTY-<key> | Get the value of a property <key>. If the value is not initialized, an empty string is returned. <key> : Poll_Time ?PROPERTY- Poll_Time |
| PROPERTY-<key>,<value> | Set the value of property <key> to <value>. This must be followed by the REINIT command to take effect. These values are not initialized by default. <key> : Poll_Time <value> : string representing the Polling interval Default - 10000, Range 1000..300000 PROPERTY-Poll_Time,10000 |
| REINIT | Re-initializes the communication link and data. Note: This command deletes any messages waiting to go out to the device. REINIT |
| ?VERSION | Query for the current version number of the Duet module. ?VERSION |
| ?XCH | Get the current tuner station. ?XCH |
| XCH-<station> | Set the current tuner station. <station> :Tuner station XCH-306.1 XCH-22 |

Table 3 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for DirecTV HR21 changes via command events. The commands supported are listed below.

PLEASE NOTE: Feedback is only provided when there is a state change. If no state change resulted from the command sent in, then no feedback will be returned.

| Command | Description |
| --- | --- |
| DEBUG-<value> | Returns the state of debugging messages in the UI module and the Comm. module. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning and info messages on 4 = set all messages on DEBUG-1 |
| FWVERSION-<version> | Returns the device’s firmware version. <version> : x.y = firmware version number FWVERSION=1.6 |
| PROPERTY-<key>,<value> | Feedback on the value of property <key>. Note: An empty string is returned if the property has no value. <key> : Poll_Time <value> : string representing the Polling interval PROPERTY-Poll_Time,10000 |
| VERSION-<version> | Reports the version number of the module. <version> : x.y.z = module version number VERSION-1.0.0 |
| XCH-<station> | Returns the current tuner station. <station> :Tuner Station XCH-306.1 XCH-22 |

Table 4 - Command Feedback Definitions

## Device Notes

- The device does not support unsolicited feedback.

- The channel feedback the device returns is the last valid tuned channel.

## Programming Notes

- At startup and when the ‘REINIT’ command is used, all values are set to default values. If these values are not initialized during the startup or re-initialization sequence, then they remain set to their default values and may be returned if a query/get command is sent. The default value for all integers is –2147483648, which is the smallest valid integer available. Boolean values default to 0/false. Input default to ‘INVALID’.

- This module implements a heartbeat to tell if the device is still connected. The heartbeat is performed by issuing a firmware query command. If no response is received, the module will assume the device has gone offline, if a response is received, the module assumes the device is present.

- Since there is no unsolicited feedback the module will be out of sync when the device is controlled via front panel or remote control.

- There is no query to get the current power state. The power state is initialized to false. The power state will be updated based on the successful response for power on/off command.

- There is no query to get the current DVR transport state. The state is updated based on the successful response for the DVR command sent to the device.

- The module will poll the device to get the current channel/station. The default poll time is 10 seconds.

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.  This is the ‘PASSTHRU-‘ command, which allows protocol strings to be passed through the module.  The device-specific protocol must be known in order to use this feature.

As an example, to send the remote control command ‘GUIDE’ the command that the protocol requires is FAH,A5H, 00H, 01H, D3H.    In this case, the following string should be sent from the UI code

send_command vdvDevice, "'PASSTHRU-',$FA,$A5,$00,$01,$D3"

The reason to use ‘PASSTHRU-‘ instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the ‘PASSTHRU-‘ command like checksum calculation.)

### Responses from the device

The module will automatically interpret replies from the device and pass these on to the application code according to the documented API.  Some device replies may not be passed on to the application code.

To see all replies from the device unfiltered by the module, enable PASSBACK and use a DATA_EVENT with a string handler in the UI code.  Again, the device-specific protocol must be known in order to interpret these responses.  Even when PASSBACK is enabled, the module will still interpret device responses according to the standard API.